|  | INTERNATIONAL AMATEUR RADIO UNION REGION 3 SEVENTEENTH REGIONAL CONFERENCE |  |
| --- | --- | --- |

10 – 14 September 2018	Seoul, Korea

Document No. 18/XVII/024

Agenda Item: 11.6

Source: IARU Satellite Advisor

History

The 15-meter band 21.0-21.45 MHz, was allocated to the amateur and amateur-satellite services by WARC-1979.  It has been used by amateur radio satellites since 1987 and has proven very helpful for propagation research and DXing. Most of this use has been for linear transponder uplinks, in which the user transmits CW or SSB on 15 meters, retransmitted by the satellite on another band.

| Satellite | Launched | Frequency | Type |
| --- | --- | --- | --- |
| RS-10 | 1987 | 21.160-21.200 | Uplink |
| RS-11 | 1987 | 21.210-21.250 | Uplink |
| RS-12 | 1991 | 21.210-21.250 | Uplink |
| RS-13 | 1991 | 21.260-21.300 | Uplink |
| AO-40 | 2000 | 21.210-21.250 | Uplink |

Recent coordination requests

In 2016 the IARU Satellite Frequency Coordination panel received a coordination request for HFSAT, a USA HF transponder with an uplink in the 15 m band and a downlink in the 10 m band. For 10 m the regional band plans show where amateur satellite can be accommodated. For 15 m there is not such a provision although the ITU radio regulations allow both amateur and .amateur satellite. After consulting the IARU Regions it was agreed to allow uplink traffic in the upper part of the 15 m band.

In a formal sense no coordination was done. The sentence used in the coordination letter was: "It is understood that terrestrial amateur stations will access the transponder in the frequency band 21.385-21.415 MHz using SSB-mode”

In 2018 a similar request was received for CAS-5A, A Chinese satellite that plans to use two tranponders with uplinks in the 15 meter band

21.400 with 30 kHz BW (downlink on 10m)

21.435 with 15 kHz BW (downlink on 70 cm)

Again the IARU Regions were consulted and a coordination letter showing a similar sentence as used for HFSAT will be issued.

Proposal

Rather than asking for the Regions for permission every time a request for use of the 15 m band is submitted, the Satellite Frequency Coordination Panel prefers that the Regional bandplans will be modified to allow amateur satellite stations in all or part of 15 meters.

The panel sees three options:

1. An exclusive satellite part in the 15 m band similar to 10m.The panel doesn’t think there is enough spectrum for this option.

2. Allow amateur satellite stations in the full 15 m band. This option gives optimal flexibility. The use of different modes would follow the planning for terrestrial stations.

3. Allow amateur satellite stations in part of the 15 m band.

____________