|  | INTERNATIONAL AMATEUR RADIO UNION REGION 3 SEVENTEENTH REGIONAL CONFERENCE |  |
| --- | --- | --- |

10 – 14 September 2018	Seoul, Korea

Document No. 18/XVII/035

Agenda Item: 11.5

Interim IARU Region 3 Band Plan

(Information input by Secretary)

The tentative IARU Region 3 Band Plan was circulated among Member Societies as Circular No. G/02/17T dated 15th September 2017.

In response to this Circular, the Secretary received few modification request and comments. Modification request was to add FT8 frequency in the footnote of each band.

Comments were on APRS common frequency discussed in the 16th Conference and proceedings after that.

Please find attached the Interim IARU Region 3 Band Plan including FT8 frequencies and some more editorial modifications.

The intention of this input is to provide a reference for the discussion on IARU Region 3 Band Plan in the 17th Conference.

Attachment: Interim IARU Region 3 Band Plan (Revision 1)

____________________

IARU Region 3 Band Plan Using a New Format

Agreed by IARU Region 3 Directors

5th September 2017

For consideration by

IARU Region 3 Member Societies and

IARU Regions 1 and 2

1.  Introduction

At the 16th IARU Region 3 Conference the following Recommendation was adopted.

- Recommendation AI 11.5/XVI That the IARU Region 3 Band Plan is amended as set out in Annex 1 to this report. The Directors are requested to consider how the format of the Region 3 Band Plan can be changed in future to be similar to that of Regions 1 and 2.

To respond to this Recommendation the Region 3 Band Plan using a new format was drafted for the Region 3 Societies consideration.

2.  Proposed Region 3 Band Plan Using a New Format

The formats of the Regions 1 and 2 Band Plans are similar and the band plans are comprised of the four columns which are Frequencies, Maximum Bandwidth/BW, Mode and Usage or Applications and observations.

The Region 3 Band Plan by using a format similar to the formats of Regions 1 or 2 is shown in Attachment.

In changing the format of the current Region 3 Band Plan the following are taken into account.

- Some footnotes of the current band plans are deleted because the contents of the footnotes are included in the Tables of the new format.

- In the current band plan the portion of the band 7.2-7.3 MHz is indicated as secondary, but the portion is not allocated to the amateur service in Region 3 in the Radio Regulations. Therefore, the portion and the relevant footnote need to be deleted.

- In the current band plan the portion of the band 440-450 MHz is indicated as secondary, but in Region 3 the portion is allocated to the amateur service only in Australia and Philippines in footnote 5.270 of the Radio Regulations. Therefore the portion needs to be deleted.

- The allocation in the band 1240-1300 MHz is secondary, but no indication of secondary is made in the current band plan. To avoid misunderstanding the indication of primary or secondary is made in all band Tables of the new format.

- The revised plan has been split into two parts, Part 1 covers LF to UHF and Part 2 covers SHF to EHF.

Attachment

Interim BAND PLANS Using a New Format

IARU REGION 3

Introductory Notes

The IARU Region 3 Band Plans are revised at the triennial IARU Region 3 Conferences.

This version was adopted at the IARU R3 16th Conference, Bali, Indonesia 2015.

Basic Principles

The Basic Principles underlying the IARU Region 3 Band Plans are:

(1) In all cases of conflict between a band plan and the national regulations of a country, the latter shall prevail. However, it is not recommended to use frequencies outside of the band plan for the Amateur Satellite Service and it should be noted that the IARU cannot coordinate the amateur satellite usage of frequencies outside of the band plan.

(2) Nothing in these band plans shall be construed as prohibiting different national arrangements, provided that harmful interference is not caused to stations in countries operating in accordance with the regional band plan.

(3) Notwithstanding item (2) above, Member Societies of IARU Region 3 are strongly urged to use these regional band plans as a basis for their national band plans.

Definitions

The “Definition”/ “Abbreviations” used are:

All modes:	CW, SSB and those modes listed as Centres of Activity, plus AM (Consideration should be given to adjacent channel users).

CoA:	“Centre of Activity” frequency (IARU recommended)

Digi modes:	Any digital mode used within the appropriate bandwidth, for example RTTY, Packet, PSK, MT63, Olivia, ALE etc.  Some common usage frequencies have been identified in foot notes as appropriate.

EME:	Earth-Moon-Earth, Meteor Scatter, Auroral Scatter and other weak-signal modes

DX:	Intercontinental communications

Image modes:	Any analogue or digital image modes within the appropriate bandwidth, for example SSTV and FAX generally not exceeding 6 kHz.

Narrowband modes:

Narrow bandwidth modes including CW, RTTY, PSK, JT65, WSPR, FT8, SIM31 and modes with similar bandwidth not exceeding 500Hz.

Phone:	Phone operation includes, SSB, SSTV, FAX, FM and Digital Voice modes with similar bandwidth not exceeding 6 kHz. 6m and above bandwidth not exceeding 25 kHz

Satellite:	This segment should be kept clear of other operating modes.

WB:	Wide bandwidth modes including FM.

Contests

Contests should be restricted to 160, 80, 40, 20, 15 and 10m.

Individual Band Plans

Part One: Low Frequency (LF) – Ultra High Frequency (UHF)

LF

2200m Band:

| Frequencies (kHz) | BW (Hz) | Preferred Mode and Generic Usage | ITU Status |
| --- | --- | --- | --- |
| 135.7-137.8 | 500 | CW, QRSS and digi modes | Secondary Amateur |

Footnote:  	WSPR 136.0 kHz, JT65 136.13 kHz

MF

630m Band:

| Frequencies (kHz) | BW (Hz) | Preferred Mode and Generic Usage | ITU R3 Status |
| --- | --- | --- | --- |
| 472-479 | 500 | CW and digi modes | Secondary Amateur |

Footnote: 	WSPR 474.2 kHz, JT65 474.2 kHz

160m Band

| Frequencies (kHz) | BW (Hz) | Preferred Mode and Generic Usage |  | ITU R3 Status |  |
| --- | --- | --- | --- | --- | --- |
| 1800 – 1830 | 200 | CW |  | Primary AMATEUR |  |
| 1830 - 1840 | 500 | CW and Narrowband modes 1836 kHz - QRP CoA |  |  |  |
| 1840 - 2000 | 2700 |  | All modes |  |

Footnote:  	WSPR 1836.6 kHz, JT65 1838 kHz; FT8 1840 kHz

HF

80m Band

| Frequencies (kHz) | BW (Hz) | Preferred Mode and Generic Usage |  | ITU R3 Status |  |
| --- | --- | --- | --- | --- | --- |
| 3500 - 3510 | 200 | CW, DX |  | Primary AMATEUR |  |
| 3510 - 3535 | 200 | CW |  |  |  |
| 3535 - 3600 | 2700 | CW, 3560 kHz - QRP CoA, Phone, digi modes |  |  |  |
| 3600 - 3775 | 2700 | CW, Digi modes, Phone 3.600: Emergency CoA (all modes ± 5 kHz) 3690 kHz - Digital Voice CoA 3690 kHz – SSB QRP CoA 37350 kHz – Image CoA |  |  |  |
| 3775 - 3800 | 2700 | CW, Phone, DX |  |  |  |
| 3800 - 3900 | 2700 |  | CW, Phone, Digi modes 3795- DX Phone CoA 3845 kHz – Image CoA |  |

Footnote:   	WSPR 3592.6 kHz, JT65 3570 kHz; FT8 3573 kHz

60m Band

| Frequencies (kHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- |
| 5351.5 - 5354 | 500 | CW, Narrowband modes – digi modes | Secondary Amateur |
| 5354 - 5366 | 2700 | All modes – USB for phone operation |  |
| 5366 – 5366.5 | See Footnote | Weak signal modes |  |

Footnote:  Maximum bandwidth suggested 20Hz

40m Band

| Frequencies (kHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- |
| 7000 - 7030 | 200 | CW | Primary AMATEUR |
| 7030 -7040 | 2700 | CW, 7030 kHz – QRP CoA, Narrowband modes, Phone |  |
| 7040 – 7060 | 2700 | All modes Narrowband modes, Phone 7070 kHz – Digital Voice CoA, 7090 kHz – SSB QRP CoA |  |
| 7060 - 7100 | 2700 | All modes, 7095 – DX Phone CoA |  |
| 7100 - 7200 | 2700 | All modes, 7110 kHz - Emergency CoA (all modes ± 5 kHz) 7165 kHz – Image CoA |  |

Footnote: WSPR 7038.6.0 kHz, JT65 7076 kHz; FT8 7074 kHz; RTTY DX 7040 kHz

30m Band

| Frequencies (kHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- |
| 10110 - 10130 | 200 | CW 10116 kHz – QRP CoA | Secondary Amateur |
| 10130 - 10150 | 500 | CW, Narrowband –digi modes |  |

Footnote: (1) Some Administrations allow Phone in the segment 10115-10140 kHz.

(2) WSPR 10138.6 kHz; JT65 10138 kHz; FT8 10136 kHz

20m Band

| Frequencies (kHz) | BW (Hz) | Preferred Mode and Generic Usage | ITU R3 Status |
| --- | --- | --- | --- |
| 14000 - 14070 | 200 | CW, 14055 kHz – QRS CoA, 14060 kHz – QRP CoA | Primary AMATEUR |
| 14070 - 14110 | 500 | CW, Narrowband modes 14100 kHz - International Beacon, 500 Hz guard band for beacons |  |
| 14100 - 14112 | 2700 | CW, Narrowband – digi modes, Phone |  |
| 14112 - 14350 | 2700 | CW, Phone, 14.300 kHz - Global Emergency CoA (all modes ± 5 kHz), 14130 kHz – Digital Voice CoA, 14195 kHz DX Phone CoA, 14230 kHz – Image CoA, 14285 kHz – SSB QRP CoA |  |

Footnote: WSPR 14095.6 kHz, JT65 14076.0 kHz; FT8 14074 kHz

17m Band

| Frequencies (kHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- |
| 18068 - 18095 | 200 | CW, 18086 kHz – QRP CoA | Primary AMATEUR |
| 18095-18110 | 2700 | CW, Narrowband –digi modes |  |
| 18110-18120 | 2700 | CW, Narrowband, Phone, 18.110 kHz - International Beacon, 500 Hz guard band for beacons |  |
| 18120-18168 | 2700 | CW, Phone, 18130 kHz SSB QRP CoA 18160 kHz – Digital Voice CoA 18160kHz - Emergency CoA (all modes ± 5 kHz) |  |

Foot note: WSPR 18104.6 kHz, JT65 18102.0 kHz; FT8 18100 kHz

15m Band

| Frequencies (MHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- |
| 21000 - 21070 | 200 | CW, 21055 kHz – QRS CoA 21060 kHz – QRP CoA | Primary AMATEUR |
| 21070 - 21125 | -500 | CW, Narrowband modes |  |
| 21125 - 21150 | 2700 | CW, Narrowband modes, DX, Phone 21150 kHz - International Beacon, 500 Hz guard band for beacons |  |
| 21150 - 21450 | 2700 | CW, Phone, 21180 kHz – Digital Voice CoA, 21295 kHz – DX Phone CoA 21340 kHz – Image CoA 21360 kHz - Global Emergency CoA (all modes ± 5 kHz) |  |

Foot note: WSPR 21.0946 kHz, JT65 21076.0 kHz; FT8 21074 kHz

12m Band

| Frequencies (MHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- |
| 24890 - 24915 | 200 | CW, 24906 kHz – QRP CoA | Primary AMATEUR |
| 24915 - 24930 | 500 | CW, Narrowband modes |  |
| 24930 - 24940 | 2700 | CW, Narrowband modes, DX, Phone 24930 kHz - International Beacon, 500 Hz guard band for beacons, 24950 kHz – SSB QRP CoA, 24960 kHz – Digital CoA |  |
| 24940 - 24990 | 2700 | CW, Phone |  |

Foot note: WSPR 24924.6 kHz, JT65 24917.0 kHz; FT8 24915 kHz

10m Band

| Frequencies (kHz) | BW (Hz) |  | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- | --- |
| 28000 - 28070 | 200 |  | CW, 28055 kHz – QRS CoA | Primary AMATEUR |
| 28070 - 28150 | -500 |  | CW, Narrowband modes |  |
| 28150 - 28190 | -500 |  | CW, Narrowband modes, DX, |  |
| 28190 - 28201 | 200 |  | Beacons 28200 kHz - International Beacon 500 Hz guard band for beacons. |  |
| 28201 - 28225 | 200 |  | Beacons - Continuous duty |  |
| 28225 - 29100 | 2700 |  | All modes 28330 kHz – Digital Voice CoA, 28360 kHz – QRP CoA, 28495 kHz – DX CoA, 28680 kHz - Image CoA |  |
| 29100 – 29300 | 6000 |  | All modes |  |
| 29300 - 29510 | 6000 | Satellite – up link and down link |  | Primary AMATEUR-SATELITE |
| 29510 – 29590 | 6000 | All modes FM repeater input |  | Primary AMATEUR |
| 29520 – 29620 | 6000 | All Modes, FM Simplex, 29600 kHz – International Calling Channel |  |  |

| 29620 – 29700 | 6000 | All Modes FM Repeater output |  |
| --- | --- | --- | --- |

Foot note: WSPR 28124.6 kHz, JT65 28076.0 kHz; FT8 28074 kHz

VHF – VERY HIGH FREQUENCIES

6m Band

| Frequencies (MHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- |
| 50-50.1 | 200 | CW Beacons: International beacons 50.020-50.080 MHz | Primary AMATEUR |
| 50.1-50.5 | 2700 | CW, Phone, Narrowband, digi modes. 50.110 MHz – DX CoA |  |
| 50.5-54 | 6000 | All modes, WB |  |

Foot note: WSPR 50.293.7 MHz, JT65 50.276 MHz; FT8 50.313 MHz

2m Band

| Frequencies (MHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- |
| 144-144.025 | 2700 | Narrowband – digi modes (Note 1) The satellite segment should be kept clear of all terrestrial operation. | Primary AMATEUR-SATELLITE, |
| 144.025-144.035 |  | EME – weak signal | Primary AMATEUR |
| 144.035-145.8 | 6000 | All modes 144.1 MHz – DX CoA |  |
| 145.8-146 | 6000 | All modes Satellites The satellite segment should be kept clear of all terrestrial operation. | Primary AMATEUR-SATELLITE |
| 146-148 | 6000 | All Modes | Primary AMATEUR |

Footnote: (1) Designers and operators of satellites using 144.0-144.025 MHz shall not 
transmit below 144.0025 MHz so that a necessary guard band is provided at the bottom band edge.

(2) WSPR 144.4890 MHz

(3) 144.800 MHz suggested APRS Spot Frequency

(4) 144.100 DX calling frequency

UHF – ULTRA HIGH FREQUENCIES

70cm Band

| Frequencies (MHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- |
| 430-431.9 | 6000 | All Modes | Secondary Amateur |
| 431.9-432.240 | 500 | EME Weak signal |  |
| 432.240-435 | 6000 | All modes |  |
| 435-438 | 6000 | Satellites - All modes | Secondary Amateur Satellite Service |
| 438-440 | 6000 | All modes, | Secondary Amateur |

23cm Band

| Frequencies (MHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- |
| 1240-1260 |  | All modes | Secondary Amateur |
| 1260-1270 |  | Satellites; all modes limited to the Earth-to-space direction | Secondary Amateur-satellite: |
| 1270-1296 |  | All modes | Secondary Amateur |
| 1296-1297 | 2700 | EME Weak signal, SSB, Digi modes | Secondary Amateur |
| 1297-1300 |  | All modes |  |

Footnote: Societies should consult with the amateur satellite community for proposed satellite operating frequencies before deciding local band plans above 1300 MHz.

13cm band

| Frequencies (MHz) | BW (Hz) | Preferred Mode and Generic us age | ITU R3 Status |  |
| --- | --- | --- | --- | --- |
| 2300-2303.750 |  | All modes | Amateur Secondary |  |
| 2303.750-2304 | 2700 | Weak signal. CW, Phone, Digi modes |  |  |
| 2304.0-2304.1 | 2700 | Weak signal, EME. CW, Phone, Digi modes |  |  |
| 2304.1-2304.3 | 2700 | Weak signal, Calling Frequency 2304.100 |  |
| 2304.3-2304.4 | 2700 | Beacons CW, Digi modes |  |
| 2304.3-2400 | 1 MHz | All Modes |  |
| 2400-2450.0 |  | Satellite All modes – Permitted on a non-interference basis | Amateur Amateur Satellite secondary |

Part 2: Super High Frequency (SHF) to Extremely High Frequency (EHF)

SHF

9cm Band

| Frequencies (MHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |  |
| --- | --- | --- | --- | --- |
| 3300-3345.8 |  | All Modes | Amateur Secondary |  |
| 3345.8-3352.5 |  | Radio Astronomy protected band. No operation |  |  |
| 3352.5-3400.0 |  | All Modes |  |  |
| 3400.0-3400.3 | 2700 | EME - Calling Frequency 3400.10MHz, Satellite. CW, Phone, Digi modes |  |
| 3400.3-3410.0 | 2700 | Weak Signal, Satellite - Non interference to adjacent segments, CW, Phone, Digi modes |  |
| 3410.0-3455.5 |  | All Modes | Amateur Secondary |
| 3455.5-3456.3 |  | Weak signal, Calling Frequency 3456.10 CW, SSB, Digi modes, NBFM |  |
| 3456.3-3457.0 | 2700 | Beacons, Digi modes |  |
| 3457.0-3500 |  | All Modes |  |

5cm Band

| Frequencies (MHz) |  | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |  |
| --- | --- | --- | --- | --- | --- |
| 5650-5670 |  |  | Satellite, All Modes, earth-to-space. | Secondary Amateur |  |
| 5670-5760 |  |  | All modes |  |  |
| 5760-5760.3 |  | 2700 | EME, weak signal, Calling Frequency 5760.1 MHz |  |  |
| 5760.3-5761 |  | 2700 | Beacons, Digi modes Beacon |  |
| 5761-5765 |  |  | All modes, Weak signal |  |
| 5765-5850 |  |  | All modes |  |
| 5830-5850 |  | All modes, Satellite, space-to-earth | Secondary Amateur, Amateur-satellite |  |
| 5850-5925 |  | All modes | Secondary Amateur |

3cm Band

| Frequencies (GHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |  |
| --- | --- | --- | --- | --- |
| 10.0-10.368 |  | All Modes | Secondary Amateur |  |
| 10.368-10.3683 | 2700 | Weak signal, Calling Frequency 10.3681 GHz | Secondary Amateur |  |
| 10.3683-10.3684 |  | Beacons, All modes |  |  |
| 10.3684 – 10.45 |  | All modes |  |
| 10.45-10.50 |  | All modes, Satellite | Secondary Amateur Amateur - satellite |

1.2cm Band

| Frequencies (GHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |  |
| --- | --- | --- | --- | --- |
| 24.0 - 24.048 |  | All Modes | Primary AMATEUR |  |
| 24.048 - 24.04875 | 2700 | All Modes, Narrow band Centre of Activity - 24.0482 GHz |  |  |
| 24.04875- 24.0488 | 2700 | Beacons. Digi modes Beacons |  |  |
| 24.0488-24.048995 | 2700 | Satellite, Beacons No restrictions on Satellite modes and bandwidth. | Secondary Amateur |
| 24.049-24050 | 2700 | Narrow band Satellite, | Secondary Amateur Amateur - satellite |
| 24.050-24.25 |  | Wide band CoA – 24.150 GHz |  |

EHF

6mm Band

| Frequencies (GHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- |
| 47.00-47.088 |  | All modes | Primary AMATEUR, AMATEUR- SATELLITE |
| 47.088-47.090 | 2700 | All modes, Narrow band CoA 47.08820 GHz, Satellite, No bandwidth Restrictions |  |
| 47.090-47.200 |  | All modes |  |

4mm Band

| Frequencies (GHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- |
| 76.0-77.5 |  | All modes Narrow band CoA 76.0322 GHz | Secondary Amateur, Amateur-satellite |
| 77.5-78.0 | 2700 | All modes Narrow band Centre of Activity 77.5002 GHz. Satellite - No bandwidth Restrictions | Primary AMATEUR, AMATEUR-SATELLITE |
| 78.0-79.0 |  | All Modes | Secondary Amateur, Amateur-satellite |
| 79.0-81.0 |  | All modes | Secondary Amateur, Amateur-satellite |

2.5mm Band

| Frequencies (GHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |
| --- | --- | --- | --- |
| 122.250-122.251 | 2700 | All modes | Amateur Secondary |
| 122.251-123.000 |  | All modes |  |

2.0mm Band

| Frequencies (GHz) | BW (Hz) | Preferred Mode and Generic usage | ITU R3 Status |  |
| --- | --- | --- | --- | --- |
| 134.0-134.928 |  | All modes Satellite | Primary AMATEUR, AMATEUR-SATELLITE |  |
| 134.928-134.930 | 2700 | Narrow band CoA 134.930 GHz |  |  |
| 134.930-136.0 |  | All modes |  |  |
| 136.0-141.00 |  | All modes | Secondary Amateur, Amateur-satellite |

1.0mm band

| Frequencies (GHz) | BW (Hz) |  | Preferred Mode and Generic usage | ITU R3 Status |  |
| --- | --- | --- | --- | --- | --- |
| 241.0-248.0 |  |  | All modes | Secondary Amateur, Amateur-satellite |  |
| 248.0-248.01 |  |  | All modes Satellite and narrow band. |  |  |
| 248.01-250.0 |  | All modes |  | Primary AMATEUR, AMATEUR-SATELLITE |

275GHz to 3000GHz

The ITU has not allocated this segment to any radio service. Requests to use this segment should be directed to the radiocommunications regulator in their country.

---0---